# MGX ARC USB Ethernet setup

## 1. Check interfaces

Run:

ip link

Example from the attached system:

- First MAC: ba:1f:69:f2:1f:e5 on enxba1f69f21fe5.

- Second MAC: 9c:69:d3:32:16:d8 on enx9c69d33216d8.

## 2. Create or update the netplan YAML

Run:

sudo nano /etc/netplan/99-usbeth.yaml

Paste this and replace the MAC address with the first MAC from ip link:

network:

version: 2

renderer: networkd

ethernets:

usbeth0:

match:

macaddress: "REPLACE_WITH_FIRST_MAC"

set-name: usbeth0

dhcp4: true

dhcp6: false

optional: true

dhcp4-overrides:

route-metric: 100

## 3. Apply the config

Run:

sudo chmod 600 /etc/netplan/99-usbeth.yaml

sudo netplan try

sudo netplan apply

## 4. Verify IP

Run:

ip -4 addr show usbeth0

## 5. If the first MAC does not work

Update the same file and replace the MAC address with the second enx... MAC from ip link, then run again:

sudo netplan try

sudo netplan apply

ip -4 addr show usbeth0