To Flash BMC:

We are using System 6 to flash BMC but can copy to any system. The folder is under folder:

lab@vr-l117-lt-75:~/Bringup_FW/BMC_FW$

File name: MGX_ARC_26061201_dev_debug_scp_image_new_IOBoard.bin

Copy this file to BMC on any system to flash if you want to.

Run cmd:

lab@vr-l117-lt-75:~/Bringup_FW/BMC_FW$ scp MGX_ARC_26052701_dev_debug_scp_image.bin root@”BMC_IP”:/run/initramfs/image-bmc

Verify md5sum on both sides

[host] md5sum MGX_ARC_26052701_dev_debug_scp_image.bin

[bmc]  md5sum /run/initramfs/image-bmc

-------

scp MGX_ARC_26061201_dev_debug_scp_image_new_IOBoard.bin root@10.96.113.203.117:/run/initramfs/image-bmc

ssh-keygen -f "/home.local/lab/.ssh/known_hosts" -R "10.137.203.117"

----------

To Update:

[bmc]  /run/initramfs/update   #will detect BMC image at /run/initramfs/image-bmc

After finish, cold power system then check IP to see if the same.

To flash CX8:

Copy two files to local BMC from system6 under CX8_QP

fw-ConnectX8-rel-40_97_5427-cx8_P4180_MGX_ARC_QP_Ax.bin

mstflint

Steps to flash Cx8:

Powerctrl.sh power_on

for eid in 60 70; do mctp-client eid $eid type pci data 10 DE 80 02 00 10 00 05 00 00 00 01 00 01 00 00; done && sleep 0.5 && gpioset `gpiofind S_MCU1_RECOVERY_N-O`=0 && gpioset `gpiofind S_MCU2_RECOVERY_N-O`=0 && gpioset `gpiofind S_FNP_CX2_L-O`=0 && gpioset `gpiofind S_FNP_CX1_L-O`=0 && gpioset `gpiofind S_FNP_CX0_L-O`=0 && powerctrl.sh power_cycle

i2cdetect -y 26

i2cdetect-y 28

i2cdetect -y 30

chmod +x /tmp/mstflint

bus=26

/tmp/mstflint -d /dev/i2c-$bus --i2c_secondary 0x48 -i /tmp/fw-ConnectX8-rel-40_97_5427-cx8_P4180_MGX_ARC_QP_Ax.bin -ocr -nofs --ignore_dev_data --no_flash_verify burn

bus=28

Same cmd as above for flash.

bus=30

Same flash cmd as above.

Cold Power cycle system

To check to see if CX8 list out:

Flash MCU

How to update MCU:

root@mgx-arc:~# mctp-list-eps

Transport| EID|                                             UUID|Supported MCTP Types

USB| 100|           "00000000-0000-0000-0000-000000000000"| 0

USB|  12|           "c01dc250-5678-11ed-9b6a-0242ac120002"| 126

USB|  14|           "f72d6f60-5675-11ed-9b6a-0242ac120002"| 1 5 126 127

SPI|  30|           "00000000-0000-0000-0000-000000000000"| 0

SPI|  50|           "06030313-1914-1e22-99e7-cf200ed6ec80"| 1 5 126 127

USB|  80|           "64da3af3-12f7-6c51-ae9c-2fecf54a270b"| 1 5 126 127

root@mgx-arc:~#

------------

root@mgx-arc:~# lsusb

Bus 001 Device 001: ID 1d6b:0002 Linux 6.12.59-f0e329a-gf0e329aae6b1 ehci_hcd EHCI Host Controller

Bus 001 Device 002: ID 0424:2514

Bus 001 Device 003: ID 0955:ffff

Bus 001 Device 004: ID 0424:2514

Bus 001 Device 005: ID 0424:2514

Bus 001 Device 006: ID 1fc9:0158 NXP SEMICONDUCTOR INC. USB COMPOSITE DEVICE

Bus 001 Device 007: ID 0955:cf11

Bus 001 Device 008: ID 1fc9:0158 NXP SEMICONDUCTOR INC. USB COMPOSITE DEVICE

-------------

gpioset $(gpiofind S_MCU1_RST_N-O)=0

gpioset $(gpiofind S_MCU1_RECOVERY_N-O)=0

gpioset $(gpiofind S_MCU2_RST_N-O)=0

gpioset $(gpiofind S_MCU2_RECOVERY_N-O)=0

[ 0 ] root@mgx-arc:~# lsusb   # unflashed MCUs gone now

Bus 001 Device 001: ID 1d6b:0002 Linux 6.12.59-f0e329a-gf0e329aae6b1 ehci_hcd EHCI Host Controller

Bus 001 Device 002: ID 0424:2514

Bus 001 Device 003: ID 0955:ffff

Bus 001 Device 004: ID 0424:2514

Bus 001 Device 005: ID 0424:2514

Bus 001 Device 007: ID 0955:cf11

----------

gpioset $(gpiofind S_MCU1_RST_N-O)=1

blhost --usb-id 1fc9:0158 -- flash-erase-all 0

-----------

>>Inject command 'flash-erase-all'

>>Successful generic response to command 'flash-erase-all'

>>Response status = 0 (0x0) Success.

-----------

blhost --usb-id 1fc9:0158 -- write-memory 0 /tmp/mcxn236-mcufw-p4180-0036.00.0283.0008-rel-debug.bin

-----------

gpioset $(gpiofind S_MCU1_RST_N-O)=0

gpioset $(gpiofind S_MCU1_RECOVERY_N-O)=1

gpioset $(gpiofind S_MCU1_RST_N-O)=1

gpioset $(gpiofind S_MCU2_RST_N-O)=1

-----------

blhost --usb-id 1fc9:0158 -- flash-erase-all 0

-----------

blhost --usb-id 1fc9:0158 -- write-memory 0 /tmp/mcxn236-mcufw-p4180-0036.00.0283.0008-rel-debug.bin

------------

gpioset $(gpiofind S_MCU2_RST_N-O)=0

gpioset $(gpiofind S_MCU2_RECOVERY_N-O)=1

gpioset $(gpiofind S_MCU2_RST_N-O)=1